QUOTATION

Quotation No. : VF/QUO/26-27/152					                 Ref. No.

To,	                                                                      				    Date:  07/07/2026

M/S Himangi Foods Pvt. Ltd.

52, Uptron Estate,

Panki Industrial Area,

Kanpur – 208022

Kind Att.

# Dear Sir,

# We appreciate your interest in our products and are delighted to provide the following price.

| # Sr. No. | # Descriptions | # Vehicle | # Qty. | # Rate (₹) |
| --- | --- | --- | --- | --- |
| # 1. | # 42 m3 Bulker for Wheat Flour # Hopper Type . |  | # 1 No. | # ₹ 11,55,300/- |
| # 2 | # Vibrating Motors 2 Nos. |  |  | # ₹ 75,000/- |
| # 3 | # Aluminium platform with SS Support, SS body Butterfly valve |  |  | # ₹ 1,05,000/- |
|  | # Unladed weight – Approx. 4590 kg. | # GST | # 18% | # ₹ 2,40,354/- |
|  |  | # Grand Total |  | # ₹ 15,75,654/- |

Specifications:

- Model No:	VFB-32

- Type:		32 M³ Bulker for Flour

- Construction:  Self Supported Pressure vessel made of High Tensile Steel

    - Shell: 		High Tensile Steel

    - Dish End: 	IS 2062, 5 mm Thick

    - Leg Supports:	IS 2062, 5 mm Thick

    - Compartments: Single

- Dimensions:	Overall Length: 		9270 mm

Width: 			2600 mm

Height above the chassis:	3050 mm

Height fro Ground level: 	4250 mm

- Pipeline: 	3” connection for air inlet with pressure relief valve and

5” connection for material out let at rear end of the tanker

All pipeline are in SS 304.

- Discharge: 	By means of pneumatic pressure with dry and oil free air

Material outlet DN 125 with butterfly valve under 1.95 kg/cm² pressure.

Inside of tanker an aeration cloth made of synthetic material is

used for better fluidization and long life of cloth.

- Manhole: 	450 mm, 4 Nos. Manhole in S.S 304

  - Safety:		Pressure relief valve on vessel, 1” ball valve on inlet pipe to   maintain

Pressure, one 3” connection with butter fly valve SS Flap with CI body on vessel.

- Catwalk: 	M.S. Catwalk on both the sides, 300 mm wide with handrail and ladder.

- Paint: 	Bulker is completely shot blasted then painted with two layers of heavy

duty primer then two coats of synthetic enamel paint, inside two coats

of food grade epoxy paint. Paint without lettering.

- Accessories: 	Mud guards, 4” pressure gauge with limit marker.

Note : - 30FT Chassis Length required

General  Sales Terms:

- Subject to Chhapi Jurisdiction

- Terms of Del : Ex- works Chhapi, Banaskantha, North Gujarat

- Payment terms: 30 % Advance and balance against proforma invoice.

- Delivery time: 60 days from receipt of confirmed order with advance.

- Warranty: Our product will be covered warranty for 12 months against defective material and workmanship.  We are not responsible against mishandling of equipments and consequential damage during accident & in such case any replacement of spares and labour shall be on chargeable basis.

No warranty on hydraulic equipments and parts. Brought out items have their own company standard warranty which will be provided by us.

- Major maintenance will be done at our works. Owner has to send the vehicle by own and all the expenses on RTO registration tax & other charges, insurance will be extra.

- If require extra Cabin Supports, extra “U” Bolt, Flat-belt, Chassis Support & Trucks Bumper and male coupler will be on chargeable basis.

- If any other connection and flanges required will be on chargeable basis.

- Validity: Up to 20 days.

We believe and hope that our affordable rate and other details will be acceptable to you.

We anticipate accepting your esteemed buy request.

Thanking you.

Yours faithfully,

Mr. Nikhil Panchal

Director