QUOTATION

Quotation No. : VF/QUO/26-27/154                                                                Ref. No.  VF/

To,									                Date: - 07/07/2026

M/s. Sri Vishnu Logistics

P. Bala Krishna Prasad Raju,

Flat No. 202, Devi Homes,

Plot No. 255, Road No. 34, Survey of India Colony,

Ayyappa Society, Madhapur,

Hydrabad - 500081

Kind att.:

Dear Sir,

# We appreciate your interest in our products and are delighted to provide the following price.

| # Sr. No. | # Descriptions | # Vehicle | # Qty. | # Rate (₹) |
| --- | --- | --- | --- | --- |
| # 1. | # 45 m3 Triple Axle Bulker Semi Trailer without Tyres | # 4 x 2, # 6 Wheeler | # 1 No. | # ₹ 15,55,085/- |
|  | # Approx. weight - 8300 Kg | # GST | # 18% | # ₹ 2,79,915/- |
|  |  | # Grand Total |  | # ₹ 18,35,000/- |

Specifications:

- Model No:	VFBT-45T

- Type:		45 M³ Triple Axle Bulker Semi Trailer

- Construction:  Self Supported Pressure vessel made of Mild Steel

    - Shell: 		IS 2062 E350, 4 and 5 mm Thick

    - Dish End: 	IS 2062 E250, 5 mm Thick

    - Chassis & Leg support: High Tensile Steel, 5 mm Thick

Welded with adequate cross members

- Pipeline: 	3” connection for air inlet with pressure relief valve and

5” connection for material out let at rear end of the tanker

- Dimension :    Length :- 11850 mm

Height: - 4120 mm

Width:-2560 mm

- Discharge: 	By means of pneumatic pressure with dry and oil free air

Material outlet DN 125 with butterfly valve under 1.95 kg/cm² pressures.

Inside of tanker an aeration cloth made of synthetic material is

Used for better fluidization and long life of cloth.

- Manhole: 	DN 600, 3 Nos.

- Safety:		Pressure relief valve on vessel, 1” ball valve on inlet pipe to maintain

Pressure, one 3” connection with butter fly valve on vessel.

- Catwalk: 	M.S. Catwalk on one sides, 300 mm wide with handrail and ladder.

- Accessories:   Female coupler with cam locks at inlet and outlet, mud guards,

4” pressure gauge with limit marker.

- Electrical:	24 volt electrical system. 7 pin electrical connector. Parking, brake and

Indicator lights.

- Paint: 		Bulker is completely Shot Blasted and painted with two layers of anti-

corrosive Metal Primer Heavy duty – Zinc Phosphate com, two coat of synthetic paint, inside Enameled paint. Paint without lettering

- Running Gear:

- Axles:			02 Nos. York make square beam Axle with lift, Model-6625

01 No. York make square beam Axle, Model-6625

Capacity	: per axle 13.0 tons at 105 km/hr

Axle Beam	: 6 inch (Square Axle Beam)

Track width	: 1950 mm

- King Pin:		50.4mm YORK make bolted on 12 mm steel plate

- Axle Brackets and

Suspensions:          	York make air suspension Tecair 1 HD, 14MT, Capacity

2 Auto Lift Axle system.

- Tyres:			10.00xR20, 12 nos, Vikant A511 Tyres.

- Wheel Rims:		7.5” x 20”, 10 Holes, 12 Nos.  Wheels India Make

- Landing Legs:	York 2 Speed with static capacity 64 Tons and lift capacity

- Tones per pair

- Brakes:		 2 Line Air Brake System with Three Air tank  (120 ltrs),

Mechanical slack adjusters.  EBS Brake System

- Relay Valve:		RE 6 Emergency relay valve M332930 of Webco Ltd

M 22 x 1.5 Reservoir Port, M 14 x 1.5 Ports for service

M 16 x 1.5 ports for Emergency and 4 nos. Delivery

- Brake Chamber:	Type 24, Port size: M 16 x 1.5, 6 nos.

- Other Accessories: 	SUPD, Mud-guards and Bumper.

General Sales Terms:

- Subject to Chhapi Jurisdiction

- Terms of Del : Ex- works Chhapi, Banaskantha, North Gujarat

- Payment terms: 30 % Advance and balance against proforma invoice.

- In the event of cancellation of order, the advance payment won’t be refundable.

- Delivery time: 12 Weeks from the receipt of confirm order with advance.

- Warranty: Our product will be covered warranty for 12 months against defective material and workmanship.  We are not responsible against mishandling of equipments and consequential damage during accident & in such case any replacement of spares and labour shall be on chargeable basis.

No warranty on hydraulic equipments and parts. Brought out items have their own company standard warranty which will be provided by us.

- Major maintenance will be done at our works. Owner has to send the vehicle at our premises by own and all the expenses on from owners account.

- RTO registration tax & other charges, insurance will be extra.

- If require extra Cabin Supports, extra “U” Bolt, Flat-belt, Chassis Support & Trucks Bumper and male coupler will be on chargeable basis.

- If any other connection and flanges required will be on chargeable basis.

- Validity: Up to 20 days.

We believe and hope that our affordable rate and other details will be acceptable to you.

We anticipate accepting your esteemed buy request.

Thanking you.

Yours faithfully,

Mr. Nikhil Panchal

Director