QUOTATION

Quotation No. : VF/QUO/26-27/156	                                                        Ref. No.

To,										Date: - 10/07/2026

M/S. Shree Shyam Minchem

Madhusudan Sharma

Udaipur

Kind Att.Mr.Madhusudan Sharma

# Dear Sir,

# We thank you for interest shown in our products and pleased to offer pricing as under.

| # Sr. No. | # Descriptions | # Qty. | # Rate (₹) | # Total (₹) |
| --- | --- | --- | --- | --- |
| # 1. | # 34’ x 5’ Side Wall Semi Trailer without Tyres and rims # Made from - High Tensile Steel | # 1 Nos. | # ₹ 12,30,000/- | # ₹ 12,30,000/- |
| # Approx. weight: 7300 kg. |  | # GST (18%) |  | # ₹ 2,21,400/- |
|  |  | # Total |  | # ₹ 14,51,400/- |

## TECHNICAL SPECIFICATIONS	:

#### MODEL:		VFFT – 34T

TYPE:			34’ X 5 ft Side wall semi-trailer tridem axles

WEIGHT:		Tare		-	APPROX 7300 Kg.

Vehicle Model – Tata Signa 5532

DIMENSIONS:	Length		-	 10455 mm

Width		-	 2590 mm

Width 		-	 2395 mm (internal)

Height		-	 1520 mm (internal) (5’)

Height		-	 3000 mm from top to ground level

TRAILER SPECIFICATION:

AXLES:		02 Nos.  York make round beam Axle, Model-6625

01 No. York make round beam axle with lift, Model-6625

Capacity:	per axle 13.0 tons at 105 km/hr

Axle Beam	: 6 inch

Track Length	: 1950 mm

SUSPENSION: 	York Make 1 no. Mono Air Suspension with manual lift

axle, tandem mechanical suspension, 16 MT capacity.

CHASSIS:	02 Nos. Fabricated Beams

BEAM DETAILS:	Flanged Thickness – 8 mm and 10 mm

Web Thickness – 6 mm

CROSS MEMBERS:	Adequately reinforced pressed channels.

FLOOR:	2.75 High Tensile steel with adequate cross members

SIDE WALL:	2 mm thick 5’ Height

KING PIN:		50.4mm YORK/Jost make bolted on 12 mm steel plate

LANDING LEGS:	2 Speed with static capacity 64 Tons and lift capacity

- Tones per pair

TYRES:		NA

RIMS:			NA

BRAKES:		2 line air brake system with two air tank (80 ltrs.)

and mechanical slack adjusters.

RELAY VALVE:	RE 6 Emergency relay valve M332930 of Webco Ltd

M 22 x 1.5 Reservoir Port, M 14 x 1.5 Ports for service

M 16 x 1.5 ports for Emergency and 4 nos. Delivery

BRAKE CHAMBER:	Type 24, Port size: M 16 x 1.5, 6 nos.

ELECTRICAL:	24-volt electrical system. 7 pin electrical connector. Parking, brake and indicator lights. Rear marker and side reflectors.

PAINTING:	Chassis is completely shot blasted and then painted with two layers of anti-corrosive Metal Primer (Heavy duty – Zinc Phosphate) complete trailer spray painted with three layers of automotive paint.

SPARE WHEEL:	Spare wheel carrier only, 1 no.

SUPD:	Side under run protection device (Included).

ACCESSORIES:	Tarpaulin holder at front side, Mudguards

General Sales Terms:

- Subject to Chhapi Jurisdiction

- Terms of Del : Ex- works Chhapi, Banaskantha, North Gujarat

- Payment terms: 30% Advance with purchase order and balance against proforma invoice.

- In the event of cancellation of order, the advance payment won’t be refundable.

- Delivery time: 10 weeks from the receipt of confirm order with advance.

- Warranty: Our product will be covered warranty for 12 months against defective material and workmanship.  We are not responsible against mishandling of equipments and consequential damage during accident & in such case any replacement of spares and labour shall be on chargeable basis.

No warranty on hydraulic equipments and parts. Brought out items have their own company standard warranty which will be provided by us.

- Major maintenance will be done at our works. Owner has to send the vehicle by own and all the expenses on from owners account.

- Validity: Up to 20 days.

We trust and hope our competitive rate and other information will be up to your approval.  We now look forward to receiving your valued Purchase Order.

Thanking you.

Yours faithfully,

For, Vasant Fabricators Pvt. Ltd.

Mr. Nikhil Panchal

Director