| QUOTATION |  |  |
| --- | --- | --- |
| Quotation No. : VF/QUO/26-27/164 |  | Ref. No. |
| To, SAM TOURS & TRAVELS PRIVATE LIMITED 1-40-15, Butt Road, Guindy, Chennai, Tamil Nadu – 600 016. H.O.: Coimbatore Branches: Bangalore, Cochin |  | Date: - 13/07/2026 |
| Dear Sir, |  |  |

We thank you for interest shown in our products and pleased to offer pricing as under.

| Sr. No. | Descriptions | Vehicle | Qty. | Rate (₹) |
| --- | --- | --- | --- | --- |
| 1. | 30.5 KL Chemical Tanker Trailer Without Tyres | 4x2 | 1 no. | ₹ 23,25,000/- |
|  |  | GST | 18% | ₹ 4,18,500/- |
|  |  | Total |  | ₹ 27,43,500/- |

# Specifications:

- Model No:	VFCT-30.5T

- Type:	30.5 KL Tri Axle Chemical Tanker Semi Trailer

- Construction:  Self Supported Tanker made of Stainless Steel

    - Shell:		S.S.304L, 6 mm Thick

    - Dish End:	S.S.304L, 6 mm Thick

    - Baffle Plate:	S.S.304L, 6 mm Thick, 3 Nos.

    - Shape:		Elliptical

    - Chassis:		High Tensile steel 5 mm Thick Chassis,

Welded with adequate cross members

- Overall Dimensions: 		Width: 	2550 mm

Height: 	3750 mm

Length:           9600 mm

- Connections:	 3 nos. DN 450 Manholes, 1 no. 2” Vent with 1” Ball Valve,

2 nos. 8” material inlet.

- Discharge:	2” Outlet with CF3M ball valve

- Welding : 	All Butt Joints Plasma - TIG (Auto)

- Catwalk:	1 side SS 304 catwalk with MS hand rail both side of catwalk.

- Electrical:	24 volt electrical system. 7 pin electrical connector. Parking, brake and

Indicator lights.

- Paint: 		All MS Parts shot blasted, Tanker is completely painted with Two layers of

Anti-corrosive Metal Primer Heavy duty – Zinc Phosphate, two coats of PU paint, Min. 110 DFT.  Paint without lettering.

|  | Running Gear: |  |
| --- | --- | --- |
| 1. | Axles: 02 Nos. York Axle with pole wheel and sensor, Model 6625 01 No. York Axle, without pole wheel sensor, Model 6625, Capacity : per axle 13.0 tons at 105 km/hr Axle Beam : 6 inch (Square Axle Beam) Track Length : 1950 mm |  |
| 2. | King Pin: 50.4mm YORK make bolted on 12 mm steel plate |  |
| 3. | Suspensions: York makes air suspension Tecair 1 HD, 14MT Capacity, 2 Auto Lift Axle Systems. |  |
| 4. | Tyres: NA |  |
| 5. | Wheel Rims: 7.5 x 20, 10 Holes, 12 nos. Wheels India Make |  |

- Landing Legs:	2 Speed with static capacity 64 Tons and lift capacity 28 Tones per pair

- Brakes:	2 Line Air Brake System with three Air tank (120 ltrs),

Mechanical slack adjusters. (EBS Brake System)

- Relay Valve:	RE 6 Emergency relay valve M302930 of Wabco Ltd

M 22x 1.5 Reservoir Port, M 14 x 1.5 Ports for service M 16x 1.5 ports for Emergency and 4 nos. Delivery

## Brake Chamber:	Type 24, Port size: M 16 x 1.5, 6 nos.

## Other Accessories: Mud-guards and Bumper, Mild steel Valve Box, PTFE Gaskets, SS 304

## Fasteners.

# General Sales Terms:

- Subject to Chhapi Jurisdiction

- Terms of Del : Ex- works Chhapi,Banaskantha, North Gujarat

- Payment terms: 30 % Advance with purchase order and balance against delivery.

- In the event of cancellation of order, the advance payment won’t be refundable

- Delivery time: Delivery start from 40 days from the date of confirm order. Total 80 days required to complete 10 Nos.

- Warranty: Our product will be covered warranty for 12 months against

Defective material and workmanship. We are not responsible against mishandling of equ ipments and consequential damage during accident & in such case any replacement of spares and labour shall be on chargeable basis.

No warranty on hydraulic equipments and parts. Brought out items have their own company standard warranty which will be provided by us.

Major maintenance will be done at our works. Owner has to send the vehicle by own and all the expenses on from owners account

- RTO registration tax & other charges, insurance will be extra.

- If any other connection and flanges required will be on chargeable basis.

- Validity: Up to 20 days

We trust and hope our competitive rate and other information will be up to your approval.

We now look forward to receiving your valued Purchase Order.

Thanking you.

Yours faithfully,

Mr. Nikhil Panchal

Director