QUOTATION

Quotation No. : VF/QUO/26-27/165						Ref. No.

To,									                  Date: 13/07//2026

M/S Keith Walking Floor India

Kind Att.: Mr. Dharmendra Sakrecha – Mob No.: +91 9345961738

# Dear Sir,

# We thank you for interest shown in our products and pleased to offer pricing as under.

| # Sr. No. | # Description | # Qty. | # Basic Price (Rs) |
| --- | --- | --- | --- |
| # 1. | # 40 ft Walking Floor Container without Roof panel, floor suitable for walking floor installation and PU paint color (CSC certificate included) | # 1 No. | # ₹ 8,90,000/- |
|  |  | # GST (18%) | # ₹ 1,60,200/- |
|  |  | # Total (R/F) | # ₹ 10,50,200/- |

Standards : IS 1496- 1 , ISO 668 and ISO 1161

Frame: 	Side Bottom Rail (Outer) - 	IS 2062 E350 or Equivalent, 4.5 mm Thick

Corner Post-			IS 2062 E350 or Equivalent, 6 mm Thick

Cross Member- 		IS 2062 E350 or Equivalent, 4.5 mm Thick

End Frame- 			IS 2062 E350 or Equivalent, 4.5 mm Thick &

6 mm Threshold plate

Cross Bracings- 		25 NB pipe cross bracings with screws – 4 nos.

Panel:		Door- 				IS 2062 E350 or Equivalent, 2 mm panel

Roof- 				NA.

Side- 				IS 2062 E350 or Equivalent,

2 mm Corrugated with inner side 3 mm flat sheet.

Front- 				IS 2062 E350 or Equivalent,

2 mm corrugated with bottom side cover plat.

Corner Casting: SCW49

Door Hardware: SAEJIN SJ-13BF model OR Bloxwich BE2566 Model

Cam & Keeper: SF45A

Dimensions:

External: 		Length- Approx. 12192 mm

Width- Approx. 2438 mm

Height- Approx. 2896 mm

Internal: 		Length- Approx. 12000 mm

Width- Approx. 2352 mm

Height- Approx. 2600 mm

Door Opening: 	Width- Approx. 2340 mm

Height- Approx. 2302 mm

Ratings:

Internal Capacity: 	68.6m3

Max. Gross weight: 	32480 kgs

Tare Weight: 		Approx. 5900 kgs (without walking floor)

Max. Payload: 	26580 kgs

Stacking Test Load: 	2,13,000 kgs

Floor Strength: 	7260 kgs

Walking floor Attachments:

- 25 x 25 x 1.5 mm Thick Pipes included for slats installation

- Floor height from the bottom of corner cast 178 mm.

- Floor will be ready for walking floor installation.

Surface Protection

Surface Preparation:

All steel components, prior to forming, will be shot-blasted to a SA 2.5 standard surface by means of an automatic centrifugal shot surface cleaning machine. A weld-able primer compatible to the paint system will be applied immediately to a thickness of 10 to 15 micron to preserve the surface integrity during the assembly process. After the container is assembled it is shot-blasted again manually to clean all the welds and any other area that was contaminated during the assembly process. Slags and spatters are removed by means of grinding or needle hammers.

Paint:

The zinc content of zinc rich primer shall be no less than 65% measured by weight of solid.

Exterior:

- Solvent Based Epoxy Zinc Rich Primer to 10 mic DFT.

- Apply one coat of waterborne zinc rich primer to 20 mic. DFT.

- Apply one coat of waterborne epoxy primer to 40 mic. DFT.

- Apply one coat of Waterborne Acrylic top coat for Type–I: RAL 7033 &

- for Type –II & Type- III: RAL 5017 to 40 mic. DFT.

- Total 110 mic. DFT.

Interior:

- Solvent Based Epoxy Zinc Rich Primer to 10 mic DFT.

- Apply one coat of waterborne epoxy zinc rich primer to 20 mic. DFT.

- Apply one coat of Waterborne Acrylic top coat RAL 8035 to 50 mic. DFT.

- Total 80 mic. DFT.

Understructure:

- Solvent Based Epoxy Zinc Rich Primer to 10 mic DFT.

- Apply one coat of waterborne epoxy zinc rich primer to 20 mic. DFT.

- Waterborne undercoating (waxy or bituminous) to 200 (steel parts)

Floor: Suitable for walking floor as per Keith Guideline.

Scope of work:

- Supply of ISO box container with CSC Number and Plate

- Walking floor installation ready container

- Other supply specification as above.

- Walking floor supply and installation not in VFPL scope.

General Sales Terms:

- Subject to Chhapi Jurisdiction

- Terms of Del : Ex works Chhapi, Banaskantha, North Gujarat

- Payment terms: 35% Advance and balance against delivery.

- In the event of cancellation of order, the advance payment won’t be refundable.

- Delivery time:  3 months from receipt of Purchase order with advance payment.

- Warranty: Our product will be covered warranty for 12 months against defective material and workmanship.  We are not responsible against mishandling of equipments and consequential damage during accident & in such case any replacement of spares and labour shall be on chargeable basis.

No warranty on hydraulic equipments and parts. Brought out items have their own company standard warranty which will be provided by us.

- Validity: Up to 20 days.

We trust and hope our competitive rate and other information will be up to your approval.

We now look forward to receiving your valued Purchase Order.

Thanking you.

Yours faithfully,

Mr. Nikhil Panchal

Director