QUOTATION

Quotation No. : VF/QUO/26-27/109					          Ref. No.

M/s Vedanta Limited							Date:-12/06/2026

Jharsuguda, Odisha – 768201

Kind Att. : Ms. Shivani Yadav, Co. No. +91 9079614552

Dear Sir,

# We appreciate your interest in our products and are delighted to provide the following price.

| Sr. No. | Descriptions | Qty | Total (₹) |
| --- | --- | --- | --- |
| 1. | # 20’ ISO DRY BULK TANKER # For Transportation by Rail, Sea and Road Tanker with CSC certificate. # Designed for Dry Bulk material discharge by compressed air Model 25CBM, # With Rail Impact Test | 1 Nos. |  |
|  |  | GST |  |
|  |  | Total |  |

# Technical Specifications:

Tank Type	:	20’ HC ISO TANK CONTAINER

## Design and testing:

Tanker	:	ASME VIII Div 1 and IS 2825, where applicable

Frame	:	ISO 1496-3, ISO 6346, ISO 668 and ISO 1161.

## Rail Impact Tested

Dimensions and Thickness

Frame Dimensions	:	6058 mm long x 2438 mm width x 2896 mm height Shell Calculated Thickness	:	4 mm Taken

Shell Corrosion	:	0.5 mm

Head Corrosion	:	0.5 mm Head After Forming Min.	:	4.5 mm

Application	:	Cement

## Diagonal Difference

Roof, Bottom & Side

Diagonals	:           13 m

Front & Rear Diagonals	:           10 mm

## Rating

Tare (Nominal: +/- 3%)	:	Approx. 4300 kg Max. Payload	:	31700 kg

Max. Gross	:	36000 kg

## Stacking	:	213000 kg

Unloading Time	:           Maximum 50 minutes at pressure of 1 bar for unloading of

31000 kgs cement from container with discharge pipeline of 30 meters.

Remnant Ratio % cement     :          <= 0.7

## Pressure and Temperature Rating

Tank M.A.W.P	 :         2.2 Bar

Tank Test Pressure	 :         3.3 bar

Tank Design Temperature	 :         -19 ˚C to + 120 ˚C

Ambient Temperature	 :         -6 ˚C to 80˚C

## Material Construction	   :

Shell	  :	IS 2062 E350

Dished Ends	  :	IS 2062 E250

Frame to Shell Material	  :	E350 High Tensile Steel

Frame Work	   :	  Hollow Pipe Sections YS 310 or Higher

Corner Castings	   :	  ISO Standard 1161

Top & Bottom: Standard ISO Castings,

Radiography	   :	  Spot radiography on shell including all T joints

Dished ends Ends 100% full

Joint efficiencies 0.85 for Shell, 1.0 for Dished Ends

Surface Finish and Cleaning

Surface Finish

Shell	:	Hot Rolled Steel

Dished Ends	:	Hot Rolled Steel

Longitudinal Welds	 :	As Welded Circumferential Welds	 :	 As welded Internal Nozzle Welds	:	 As welded

Cleaning	:		On completion of fabrication the vessel’s internal surface Cleaned. Then painted with synthetic enamel paint one coat.

## Tank Fitting

Manlid	:	ø 490 mm ID (This size widely using in all the ISO tank container in India)

Quantity	:	03 nos.

Description	:	Mild steel, manlid open to 135° Max. from rear to front 6-point EN8 wing nuts, low profile

Gasket	:	EPDM Solid Rubber

## Safety Relief Valve	:	At Air Pipeline

Dimension	:	Threaded Safety relief valve 1” BSP

Quantity	:	One

Description	:	Stainless steel body, with Brass Nozzle and Disc,

Model- #004-DN25-A-2, Part No.- #VFPL-985, for more details refer catalog

## Air Pipeline

Dimension	:	3” (DN 80) and 2” (DN50)

Quantity	:	1 Side openings

Description	:	IS 1239 standard YS 210 Steel ERW B Class pipeline with Ball valves (Brass)

## Vent

Dimension	:	2” (DN50)

Quantity	:	3 Nos. Two on Top of the tank and one at air inlet Pipeline Description	:	IS 1239 standard YS 210 Steel ERW B Class pipeline with

Ball valves (Brass)

## Material discharge line	:

Dimension	:	5” (DN125)

Quantity	:	1 Side Discharge only

Description	:	SA106 Seamless Pipeline with 4 nos. Manual Butterfly

## valves ((Aluminum Dia cast body with SS disc imported Butterfly Valve)

Gasket	:	EPDM Rubber

Top Longitudinal Member :	Top longitudinal rectangular members connected the front

And rear end frame top corner castings.

Bottom Longitudinal Member:	Bottom longitudinal Pipe members connected the front

And rear end frames, above bottom corner castings.

## Walkway

Width	:	500 mm, Both sides

Material	:	Steel with Galvanized coating

Type	:	Expandable Mesh with Reinforcement

Description	:	One longitudinal walkway fitted at Both sides

## Ladder

Quantity	:	Two

Width	:	350mm

Material	:	MS

Description	:	Fitted to rear end right side and other Inside the tank.

Bolted fasteners to be tack welded to prevent theft.

## Data Plate

Quantity	:	One

Material	:	Stainless steel

Description	:	Data to code requirements indicate applicable

Position	:	Rear end left hand side.

Owner Logos	:	Supplied and fitted by manufacturer.

## Painting

Framework Treatment	:	All carbon steel frame parts will be blast cleaned in

accordance With SA 21/2

## Painting of Frame and Tank

Primer Coat	:	Epoxy zinc rich primer, 30microns DFT. Intermediate coat	:	Epoxy primer, 40 microns DFT.

Top coat	:	Poly Urathene, 50 microns DFT

Total DFT	:	120 microns DFT

Color	:	Suit to customer.

## Internal Painting of Tank

Primer Coat	:	Epoxy zinc rich primer, 30microns DFT.

Top coat	:	Synthetic Enamel, 50 microns DFT

Total DFT	:	80 microns DFT

Earthing Connections	:	At least one lug with 20 mm hole is located at near of tank Adjacent to bottom discharge.

## Nozzle Schedule

| No. | Name | Flange | Size | SCH./ Class | Thick | Qty |
| --- | --- | --- | --- | --- | --- | --- |
| N1 | Outlet Nozzle | TA HB VF 002 02 00 22 | 125 NB | SA 106/SCH 40 | 6.55 | 1 |
| N2 | Air Inlet | - | 50 NB | IS 1239 B and C Class | 3.6 | 1 |
| N3 | Aeration Inlet | - | 40 NB | IS 1239 B and C Class | 3.2 | 4 |
| N4 | Pressure Gauge | 15 NB NPT full Thread Coupling | 15 NB | 3000 Class | STD | 1 |
| N5 | Safety Valve | 25 NB BSP full Thread Coupling | 25 NB | 3000 Class | STD | 1 |
| N6 | Air Header | - | 80 NB | IS 1239 B and C Class | 4 | 1 |
| M1 | Manhole | - | 490 MM | IS 2062 E250 | 6 | 3 |

## Test and Approvals:

Frame Test as per IS 1496-3 with rail impact test and Type Approval of tank design

## Documents to be provided:

Details and GA drawings with indication of the serial number Tank Inspection report

Pneumatic Test report

Warranty Letter for 12 months

## Scope of work:

- We will make ISO tanker as per above specifications and this is standard equipment. If any other changes from the customer’s end will subject to revised rates.

- NDE or radiography as mention above included.

General Sales Terms:

- Subject to Chhapi Jurisdiction

- Terms of Del : Ex works Chhapi, Banaskantha, North Gujarat

- Payment terms: 30% advance with purchase Order and Balance against proforma invoice.

- In the event of cancellation of order, the advance payment won’t be refundable.

- Delivery time: 6 months from the date of confirm order with advance.

- Warranty: Our product will be covered warranty for 12 months against defective material and workmanship.  We are not responsible against mishandling of equipments and consequential damage during accident & in such case any replacement of spares and labour shall be on chargeable basis.

No warranty on hydraulic equipment’s and parts. Brought out items have their own company standard warranty which will be provided by us.

- Validity: Up to 20 days.

We now look forward to receiving your valued Purchase Order.

Thanking you.

Yours faithfully,

Mr. Nikhil Panchal

Director