QUOTATION

Quotation No. : VF/QUO/26-27/76						Ref. No.

To,									            Date: - 18-05-2026

# M/s Container Corporation of India Limited

# Ahmedabad

# Kind Att.: Mr. Vaibhav Shah, Mo. 8128663124

# Dear Sir,

# We appreciate your interest in our products and are delighted to provide the following price.

| # Sr. No. | # Description | # Qty. | # Basic Price (₹) |
| --- | --- | --- | --- |
| # 1. | # 26 KL ISO Tank Container # Without Insulation (CSC, IMDG, 4G, RID, Impact Test certificate) | # 1 No. |  |
|  |  | # GST (18%) |  |
|  |  | # Total |  |

1.0	Technical Specifications:

1.1	Tank Type				: 	20’ ISO full frame collar tank, Type UN Portable T7,

1.2	Design Regulations:

Tanker					: 	ASME Ⅷ Div 1 (NCS), where applicable

Frame					: 	CSC, ISO 1496/3 (ICC)

General 				: 	IMDG/ADR/RID/ UN Portable tank T7, TIR, TC,

and UIC included

1.3	Classification Society            	:   	BV

1.4	Capacity (Nominal: +/- 1%)  	:  	 26,000 Liters

1.5	Dimensions and Thickness

Frame Dimensions		     	:	6058 mm Long X 2438 mm Wide X 2591 mm high

Tank Dimensions		     	:	Ø 2414 mm (I/DA) x 6020 mm (L)

Shell Calculated Thickness  	     	:	5.84 mm

Shell Corrosion                            	:      	NA

Shell Manufacture Thickness      	:       	6.0 mm

Head Calculated Thickness         	:       	5.84 mm

Head Corrosion                     		:       	NA

Head After Forming Thickness   	:       	5.84 mm

1.6	Products                             	            : 	HSD, Motor Spirit, EBMS and General Purpose

Liquid

1.7	Rating

Tare (Nominal: +/- 3%)	:	4200 Kg

Max. Payload			:	32200 kg

Max. Gross			: 	36000 kg

Stacking Weight		:          	216000 kg

1.8	Pressure and Temperature Rating

Tank M.A.W.P                       	:           2.67 bar

Tank Test Pressure                 	:           4 bar

Tank External Pressure          	:           0.41 bar / 5.9 psi

Tank Design Temperature      	:           -40˚C to 130˚C

Ambient Temperature            	:           -10 ℃ to 65 ℃

Steam Tube M.A.W.P            	:           NA

Steam Tube Test Pressure     	:           NA

2.0 	Material of construction:

2.1	Vessel Material

Shell                                        :          SA 516 Gr 70

Dished Ends                           	:          SA 516 Gr 70

Frame to Shell 		:          IS 2062 E350 or Equivalent

Steam Tubes                           	:          NA

2.2	Baffle                                     	:           2 Sets

2.3	Vacuum Stiffening Ring

Quantity                                	:            3 off

Material                               	:           IS 2062 E350 or Equivalent

Remark                                 	:           Not Used for Steam Heating.

2.4	Framework Material            :          YST 310 or Higher

2.5	Corner Castings	           	:          To ISO 1161, Standard ISO castings.

2.6	Saddles			: 	NA.

2.7	Non-Destructive Examination:        As par ASME BPVC Section VIII-1

Radiography:              Shell	:	Spot

Ends	:	Full (100%)

Joint efficiencies:       	Shell 	: 	0.85

Ends 	: 	1.0

3.0 	Surface Finish and Cleaning

3.1 	Surface Finish

Shell                                  	:            Hot Rolled

Dished Ends                    	:            As Rolled

Longitudinal Welds         	:            As welded

Circumferential Welds    	:            As welded but with 400mm ground flush on bottom center

line

Internal Nozzle Welds     	:            As Welded

3.2 	Cleaning                           	:           On completion of fabrication, the vessel’s internal surface is

Shot blasted and cleaned. The opening points are sealed so that the tank is supplied clean and ready for use.

A Cleaning Certificate will be issued by VFPL and placed in

document holder before ex-works.

4.0 Tank Fitting:

4.1	Manlid			:	(Centre spill box)

Dimension			:	ø 500 mm ID

Quantity			:	One

Description			:	Carbon Steel, manlid open to 135 Max from rear to front.

6 points Stainless steel wing nuts, low profile

Gasket				:	PTFE

Dipstick and Bracket		: 	Not fitted.

4.2	Safety Relief Valve		:	(Centre Spill Box)

Dimension			:	2.5” (DN 65)

Quantity			:	One

Description			    :	       Stainless Steel, 3.3 bar pressure / vacuum safety

relief valve, without flame proof gauze.

Weld in flange			:	Fitted tangentially off centre line, drilled/tapped

Gasket				:	Envelope PTFE.

4.3	Air Inlet		            :	(Rear spill box)

Dimension			:	1.5” (DN40)

Quantity			:	One

Description 			:	Stainless Steel, 1.5” BSP ball valve with 1.5” BSP outlet. A

Dust cap connect outlet with a stainless steel chain.

Tapped hole and plug fitted for future fitting of a pressure

gauge. Guage guard not fitted.

Weld in flange			:	Fitted tangentially off centre line, drilled/tapped-4 X M10

on 103.5mm PCD.

Gasket			            :           Not applicable.

4.4 	Top Discharge Provision 	:         (Rear Spillbox)

Dimension                              	:          	3” (DN80)

Quantity                                  :          	One

Description                             	:          Stainless Steel,3” recessed weld-in flange complete with

blank flange. Siphon tube retaining plate fitted.

Weld-in Flange                       :          Fitted horizontally on center line. dual drilled/tapped-

6 X M12 on 168mm PCD and 4 X M16 on 160mm PCD.

Gasket                                     :         	Envelope PTFE.

4.5 	Bottom Discharge 		:

Bottom discharge	            :	(Bottom Cabinet)

Dimension			:	3” (DN80)

Quantity			:	One

Description 			:	Stainless Steel, 3” 45 high lift foot valve and 3” clamped

butterfly valves connected with 3” BSP terminal connection

and dust cap, held captive with a mild steel chain.

Weld in flange			:	Weld in flange, drilled/tapped-8 x M12 on 177.8mm PCD

Gasket				:	Envelope PTFE.

Remote Control		:	Foot valve complete with cable remote closure system.

5.0 	Other Accessories

5.1 	Spillage boxes

Quantity			:	Two

Description			:	Centre spill box contains Man lid and Safety relief

Rear spill box contains air inlet and top discharge

Material			:	Hot Dip Galvanized

Drain pipes			:	Each box is completed with DN25 PVC plastic drain tubes.

Covers	:	Hot Dip Galvanized covers fitted and

Opened to about 135 from rear to front.

Remark			:	Fittings identified with stainless steel label plates

5.2 	Bottom Cabinet

Quantity			:	One

Description			:	Bottom cabinet contains the bottom discharge assembly

Material 			:	Hot Dip Galvanized

Cover and Floor		:	Hot Dip Galvanized floor and cover are fitted. The

cover can be open > 170

Remark			:	Fittings identified with stainless steel label plates

5.3	Top Longitudinal Member	:	Top longitudinal rectangular members connected the front

And rear end frame top corner castings.

5.4	Bottom Longitudinal Member:	Bottom longitudinal rectangular members connected the

Front and rear end frames, above bottom corner castings.

5.5	Miss Stacking Pickets	:

Top 				:	Miss-stacking corner plates fitted adjacent to top corner

Casting

Bottom			:	Bottom cross-members recessed adjacent to corner castings.

5.6	Walkway

Width				:	475mm

Material			:	Hot dip galvanized

Type				:	“A” type

Description			            Two full length longitudinal walkways fitted at both sides, cut

Out under the spreader plates, each divided into three individual pieces for maintenance purpose.

Three traverse walkways fitted behind the Man lid spill box, the rear Spill box and front end frame.

5.7	Handrail			:	Provision space for future fitting on right hand side.

5.8	Ladder

Quantity			:	One

Width				:	300mm

Material 			:           Stainless Steel

Description			:	Fitted to rear end right side

Bolted fasteners to be tack welded to prevent theft.

5.9	Earth Connection

Quantity			:	One

Material 			:	Mild steel painted

Dimension			:	40 x 30 x 3mm with ø9 mm hole

Position			:	One fitted to rear bottom cross member, adjacent to the ladder.

5.10	Document Holder

Quantity			:	NA

Material			:	Plastic

Dimension			:	ø75 mm x 325mm

Position			:	Fitted to inside of the right-hand side longitudinal member,

Adjacent to the rear corner post.

5.11	Data Plate

Quantity			:	One

Material 			:	Stainless steel

Description			:	Data to code requirements indicate applicable un-Portable

instruction.

Position			:	Rear end left hand side.

5.12	Calibration Plate		:	One stainless steel calibration plate marked in cm/litres is tack

Welded to the inside neck of the central spill box.

5.13	Statutory Decals		:          Supplied and fitted by Manufacturers.

5.14	Owner Logos			:          Supplied and fitted by Manufacturer.

6.0	Painting:

6.1	Framework Treatment	:	All carbon steel frame parts will be shot blasted to Swedish

Standard SA2½

6.2	Painting of Frame

Primer Coat			:	Epoxy zinc rich primer, 50 microns min DFT.

Intermediate coat		:	Epoxy primer, 50 microns min DFT.

Top coat			:	Polyurethane, 50 microns min DFT

Total DFT 			:	150 microns min DFT

Color				:	Suit to customer as per details provided

Paint Brand			:	Esdee, Asian Paints or Equivalent

6.3	Vessel				:	Same as Frame paint specification

7.0	Test and Approvals:

CSC Certificate will be provided.

Hydro test as per design with certificate

8.0     Documents to be provided:

Details and GA drawings with indication of the serial number

Technical data sheet

Scope of work:

- We will make ISO tanker as per above specifications and this is standard equipment. If any other changes from the customer’s end will subject to revised rates.

- NDE or radiography as mention above included.

General Sales Terms:

- Subject to Chhapi Jurisdiction

- Terms of Del : Ex works Chhapi, Banaskantha, North Gujarat

- Payment terms: 30% advance with purchase order balance against proforma invoice.

- Delivery time: 5 months from receipt of drawing approval.

- Warranty: Our product will be covered warranty for 12 months against defective material and workmanship.  We are not responsible against mishandling of equipment and consequential damage during accident & in such case any replacement of spares and labour shall be on chargeable basis.

No warranty on hydraulic equipments and parts. Brought out items have their own company standard warranty which will be provided by us.

- Major maintenance will be done at our works. Owner has to send the vehicle at our premises by own and all the expenses on from owners account.

- If any other connection and flanges required will be on chargeable basis.

- Validity: Up to 60 days.

We believe and hope that our affordable rate and other details will be acceptable to you.

We anticipate accepting your esteemed buy request.

Yours faithfully,

Mr. Nikhil Panchal

Technical Director